# RobotHead — ESP32-CAM Robot Head Firmware

Camera stream + 14-servo control + status LEDs for an animatronic robot head.

## 1. What this is

Firmware for an AI-Thinker ESP32-CAM that does three things at once:

- Video — serves a web UI and MJPEG stream over WiFi.

- Servo control — drives 14 servos through a PCA9685 PWM driver (I2C), accepting target angles over UDP.

- Status LEDs — the red LED blinks in patterns that tell you what the board is doing.

The ESP32 does not decide the poses itself. It listens on UDP port 4210 and moves each servo toward the latest target angle it received.

## 2. STEP-BY-STEP — do these in order

### Step 1 — Safety check first (before anything else)

Plug only the bare ESP32-CAM into power (no PCA9685, no servos, no wires on the pin headers) and watch the temperature for 30 seconds.

- Warm (regulator chip around 60-70 °C — hot but you can keep a finger on it) = normal. The small 3-legged AMS1117 regulator always runs hot on these boards.

- Too hot to touch within a few seconds, burning, smell, or discoloration = STOP. Unplug. Go to §8 Power & overheating before continuing. Do not flash or wire anything until it cools and you've found the cause.

### Step 2 — Prepare the files

Extract RobotHead.zip. Keep all files in one folder next to RobotHead.ino — the sketch will not compile if app_httpd.cpp, camera_index.h, board_config.h or camera_pins.h are elsewhere.

### Step 3 — Install Arduino IDE + ESP32 board support

- Install Arduino IDE (1.8.x or 2.x).

- File → Preferences → Additional Boards URL: https://espressif.github.io/arduino-esp32/package_esp32_index.json

- Tools → Board → Boards Manager → install esp32 by Espressif.

### Step 4 — Install the two libraries

Sketch → Include Library → Manage Libraries, install:

- Adafruit PWM Servo Driver Library

- Adafruit BusIO (usually installed automatically as a dependency)

### Step 5 — Set your WiFi

Open RobotHead.ino, lines 46-47:

const char *ssid = "YOUR_WIFI_NAME";

const char *password = "YOUR_WIFI_PASSWORD";

Must be a 2.4 GHz network — ESP32 cannot see 5 GHz.

### Step 6 — Select board settings

| Setting | Value |
| --- | --- |
| Board | AI Thinker ESP32-CAM |
| PSRAM | Enabled |
| Partition Scheme | Huge APP (3MB No OTA) |
| Upload Speed | 115200 (raise to 921600 if upload is slow) |

### Step 7 — (Optional) Apply the 2 LED edits

Makes the red LED fast-blink while video streams. Everything works without it — the LED just stays solid. See §5.

### Step 8 — Wire for flashing

ESP32-CAM has no USB port. Use a USB-TTL adapter:

| USB-TTL | ESP32-CAM |
| --- | --- |
| 5V | 5V |
| GND | GND |
| TX | U0R (RX) |
| RX | U0T (TX) |

Then: GPIO0 → GND (puts the board in flashing mode). Keep it connected during upload only.

### Step 9 — Upload

- Select the correct Port (Tools → Port).

- Press Upload.

- If it fails: check TX/RX are not swapped, hold GPIO0→GND, press Reset, try again.

- When done: remove the GPIO0→GND jumper and press Reset.

### Step 10 — Verify it boots

- Tools → Serial Monitor, baud 115200.

- Press Reset. You should see:
WiFi connected
Camera Ready! Video: http://192.168.x.x

- Note that IP. If the red LED fast-blinks forever, camera init failed — recheck Step 6 (PSRAM must be Enabled).

### Step 11 — Open the video in a browser

On a device on the same WiFi network, open http://<the IP>. You get the control page + live stream. /stream is the raw MJPEG URL.

### Step 12 — Wire the servo driver

Power off first. Connect PCA9685 to the ESP32-CAM:

| ESP32-CAM pin | PCA9685 pin |
| --- | --- |
| 5V | V+ (servo power — use a separate strong 5V supply for the servos) |
| 3.3V | VCC (logic) |
| GPIO 13 | SDA (SERVO_SDA) |
| GPIO 14 | SCL (SERVO_SCL) |
| GND | GND (common ground required) |

Then verify PCA_ADDR in RobotHead.ino:57 matches your board's solder jumpers (0x60 in code; chip default is 0x40).

### Step 13 — Send angles (the missing piece)

Nothing in this project sends servo commands. Run a sender on your PC — this moves joint 0 to 45°:

import socket

ESP_IP = "192.168.x.x"        # the IP from Step 10

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

pose = [90] * 14

pose[0] = 45

sock.sendto(bytes([1] + pose), (ESP_IP, 4210))

Full protocol in §6. Servos should ease to the target (default 150°/s max).

### Step 14 — Set limits and save a boot pose

- Tighten per-joint limits in servoBegin() so joints can't over-rotate: jointMin[3] = 20; jointMax[3] = 160;

- Send [2] to store the current pose as the power-on pose: sock.sendto(bytes([2]), (ESP_IP, 4210)) (white LED pulses to acknowledge)

## 3. File list

| File | Purpose |
| --- | --- |
| RobotHead.ino | Main sketch: WiFi, camera init, 14-servo control, UDP listener, LED status |
| app_httpd.cpp | HTTP server: / control UI, /stream video, /capture, /control |
| camera_index.h | Gzipped web UI page (the camera control interface) |
| board_config.h | Board selection — currently AI Thinker |
| camera_pins.h | Camera pin maps for each supported board |
| partitions.csv | Custom partition table (3.75 MB app slot) |
| ci.yml | CI build matrix (arduino-cli board options) |
| README.md / README.docx | This documentation |

## 4. Wiring summary (PCA9685 servo driver)

| ESP32-CAM pin | PCA9685 pin | Note |
| --- | --- | --- |
| 5V | V+ | Servo power rail. 14 servos can draw several amps — use a dedicated 5V supply, never the ESP32 board. |
| 3.3V | VCC | Logic supply. ESP32 is 3.3V logic. |
| GPIO 13 | SDA | I2C data — SERVO_SDA |
| GPIO 14 | SCL | I2C clock — SERVO_SCL |
| GND | GND | Common ground required |

#define SERVO_SDA   13

#define SERVO_SCL   14

#define PCA_ADDR    0x60   // chip default is 0x40 - match your jumpers

#define NUM_SERVOS  14

## 5. Optional LED edits

RobotHead.ino expects two edits in app_httpd.cpp so the red LED can show "video streaming" (fast blink). Skip freely — no other feature depends on this.

- Near the top of app_httpd.cpp (after the includes, around line 25): extern volatile uint32_t lastStreamFrameMs;

- Inside the while(true) loop of stream_handler(), right after int64_t fr_end = esp_timer_get_time(); (line ~281) add: lastStreamFrameMs = millis();

## 6. UDP servo protocol — port 4210

| Packet | Meaning |
| --- | --- |
| [1, a0, a1, ... a13] | Set 14 target angles, each 0-180 (clamped to joint limits) |
| [2] | Save the current pose as the startup pose (stored in flash) |
| [3, s] | Set max speed, s = degrees per 20 ms tick (1-20) |

Behaviour:

- Newest packet wins; packets are queued and drained each cycle.

- Each servo moves toward its latest target at no more than maxStep degrees every 20 ms (default 3.0 = 150 deg/s).

- Movement never stops halfway if packets pause — it finishes to the last target.

- Joints default to full 0-180; tighten them in servoBegin() to protect the mechanics.

## 7. LED status codes

### Red LED (GPIO 33, active LOW)

| Pattern | Meaning |
| --- | --- |
| Very fast blink | Camera init failed |
| Slow blink | Connecting / WiFi lost |
| Solid | Connected, waiting |
| Fast blink | Video streaming (needs the §5 edits) |
| Double pulse | Angle feed active |
| Triple pulse | Feed stopped within the last 10 s |

### White flash LED (GPIO 4)

- Short flash at boot (self-test)

- Low glow while the angle feed is active

- Brighter pulse when a "save pose" packet arrives

## 8. Power & overheating

What's normal: on an ESP32-CAM the small 3-legged AMS1117 voltage regulator (5V → 3.3V) runs genuinely hot — about 70 °C — in normal operation. So does the ESP32 module under load and the camera sensor. Warm = fine.

What's NOT normal: anything on the board reaching a burning / too-hot-to-touch temperature within a few seconds of plugging in. That means excess current — a short or a failed part.

### Test sequence

- Unplug and let it cool (2-3 minutes).

- Strip down: remove the PCA9685, all servo wires, GPIO0 jumper, SD card and anything on the pin headers. Power the bare board only.

- Still overheats in seconds → fault on the board itself:

  - Look for solder bridges / bent pins on the camera ribbon connector and header pins.

  - Look for debris, flux residue or moisture under the board and around the camera socket.

  - Common failure: the AMS1117 fails shorted and can push 5V onto the 3.3V rail — that cooks the ESP32 and camera. With a multimeter, check the 3.3V pin: should read 3.2-3.4 V. Above ~4 V = dead regulator, board needs repair/replacement.

  - Also common: a shorted ceramic capacitor on the 3.3V rail.

- Bare board stays cool, heats only when the PCA9685/servos are connected → wiring fault:

  - V+ and VCC swapped (5V into the 3.3V logic rail will kill the driver).

  - Servo supply back-feeding into the ESP32 through the 5V pin — power servos separately, common ground only.

  - Stalled/miswired servo drawing huge current.

- Only the USB-TTL adapter chip is warm → normal for CH340/CP2102 adapters. Worry only if it is too hot to touch.

- If the board already overheated badly, don't flash it — expect possible camera/sensor damage even if it boots.

## 9. Troubleshooting

| Symptom | Likely cause |
| --- | --- |
| Red LED fast-blinks forever | Camera init failed — check board selection / PSRAM |
| Red LED slow-blinks | Wrong WiFi name/password, or 5 GHz network |
| No page opens | Note IP from Serial Monitor; PC must be on the same network |
| Upload fails | Hold GPIO0 to GND during flashing, then Reset; check TX/RX not swapped |
| Compile error about psramFound / large image | Partition Scheme not set to Huge APP (3MB No OTA) |
| PCA9685 not found (serial log) | Wiring (SDA/SCL) or wrong PCA_ADDR |
| Camera fine but servos never move | No UDP sender, wrong PCA_ADDR, V+/GND not connected, or VCC/V+ swapped |
| Board too hot to touch | §8 Power & overheating |
| Image noisy | Drop config.xclk_freq_hz from 20 MHz to 10 MHz (RobotHead.ino) |

## 10. To-do checklist

- [ ] Step 1: heat check passed (bare board only warm)

- [ ] Step 5: WiFi credentials set

- [ ] Step 6: board + PSRAM + partition selected

- [ ] Step 8-9: flashed, GPIO0 jumper removed

- [ ] Step 10: serial log shows Camera Ready! + IP

- [ ] Step 11: video opens in browser

- [ ] Step 12: PCA9685 wired, PCA_ADDR verified

- [ ] Step 13: UDP sender moves a servo

- [ ] Step 14: joint limits tightened, boot pose saved

- [ ] §5: optional LED edits applied